# City of Anna

Bid Specifications

Partial Roof Replacement – Fire Department

The City of Anna is accepting bids from qualified individuals or companies for the replacement of the flat portion of the roofing system at the Fire Department located at 101 Market Street in Anna, Illinois.

GENERAL INFORMATION:

The Anna Fire Department is located at 101 Market Street in Anna, Illinois and connects with the Anna Police Department.  The flat portion of the Anna Fire Department roof is approximately 142’ x 31’.  Please note that you are bidding to replace the flat roof portion of the Anna Fire Department ONLY.

The contractor shall be responsible for removal of the existing roof and installation of a new EPDM roofing system.  The contractor shall also be responsible to furnish all materials and perform all labor necessary for the completion of:

- To remove the existing roofing down to the decking

- To inspect the decking, and if necessary immediately report any unseen damaged areas as well as remove, repair and/or replace any damaged areas

- To install an EPDM roofing system to the manufacturer’s specifications

  - 2” Poly ISO insulation board screwed to the decking

  - Tapered edge strip to facilitate drainage

  - Fully adhered 60 mil EPDM roofing system

  - Anchored before turning up wall and covering parapet wall

  - Drip edge on parapet wall and gutter edge

- All work to be completed in a workman like manner according to standard practices

- Upon completion work site should be free of all debris and the area should be canvassed with a magnet to insure that the site is free from any loose debris

- The installation must be guaranteed for a minimum of five (5) years

- The finished roofing system must provide a minimum of a twenty (20) year manufacturer’s warranty

- Any unforeseen repairs found during the course of work that may result in an additional charge should be reported immediately for payment approval.

SITE QUESTIONS:

Questions relative to the proposed work should be directed to Gary Dahmer.  He can be contacted at the Anna City Hall located at 103 Market Street, Illinois or by telephone at (618) 833-8528.

PREVAILING WAGE:

This contract calls for the construction of a “public work”, within the meaning of the Illinois Prevailing Wage Act, 820 ILCS 130/.01 et seq. (“the Act”).  The Act requires contractors and subcontractors to pay laborers, workers and mechanics performing services on public works projects no less than the “prevailing rate of wages” (hourly case wages plus fringe benefits) in the county where the work is performed.  For information regarding current prevailing water rates, please refer to the Illinois Department of Labor’s website at http://www.state.il.us/agency/idol/rates/rates.HTM.  All contractors and subcontractors rendering services under this contract must comply with all requirements of the Act, including but not limited to, all wage, notice and record keeping duties.

LICENSE AND INSURANCE REQUIREMENTS:

The contractor shall be a Licensed Roofing Contractor in the State of Illinois. The contractor shall maintain, in full force, liability and workers compensation insurance to the standard city limitations for construction contracts not less than $1,000,000. A Certificate of Liability Insurance shall be attached hereto and become part of the contract.

RIGHT TO REFUSE BIDS:

The City retains the right to refuse any and all bids in which the City deems as unqualified or otherwise not in the best interest of the City of Anna.

BID SUBMITTAL

The Bid Form and any other required documents should be sealed in an envelope and submitted to the City of Anna, located at 103 Market Street, Anna, IL 62906 no later than 4:00 p.m. on Tuesday, September 5th, 2023.  Envelopes should be clearly marked “Bid – Roof Replacement – Fire Department” on the outside.

BID:

I, __________________________________________, the bidding contractor do hereby agree to all said terms and conditions as stated above in the Bid Specifications.  Therefore, I respectfully submit the following:

- Removal and Replacement of Roofing System – Anna Fire Department

            - $______________________

      - Proposed start date - ________________________

_________________________________		_________________________

Bidder’s Signature					Date

Company Name: ___________________________________

Address: ___________________________________

City, State, Zip: ___________________________________

Phone: ___________________________________